## By Appointment to

Her Majesty The Queen

Supply and Maintenance

of Horse Weighing Equipment

##### Horse Weigh®

Installation of Horse Weigh Pit Weighing Platforms

If you are able to plan your yard before commencement of building work it is advisable to consider where you will site the weighing platform so that when the concrete is laid the pit can be incorporated into the new concrete, and the drain can be linked into the yard drains.

If you have already a complete yard then the concrete will have to be dug out to MORE THAN the following prescribed dimensions:

Kentucky Model – 	       Badminton Model -

Length – 220.00 cms		Length – 220.00 cms

Width  - 127.0 cms	       Width –  102.50 cms

Depth -  11.50 cms					             Depth –  11.00 cms

An aluminium frame will need to be purchased at the same time as the platform; the two will be delivered together. Please advise at the time of purchase what size of horse will be using the platform, as this will dictate which size of frame will be supplied for either model. The frame will create a consistently firm edge to the concrete. Concrete can then filled to the frame edge. Please refer to photograph 1. If necessary you may fill from the edge of the frame lip back to the concrete with liquid rubber, or similar, to avoid a trip hazard. Please refer to photograph 2. For further information, please see drawing no. 140 at the end of these instructions, for the aluminium frame design and pit cross-section.

When digging the pit provision will have to be made for drainage which can be either in the middle of the pit or at one end with a slight slope towards the drain hole so that no water sits continuously in the pit.

An exit hole for the cables from the load bars will need to be made in the wall of the pit nearest to where you are going to site the indicator on the wall. It is advisable that the duct for the cables is big enough so that the cables may be removed for checking for maintenance checks. We strongly advise that these cables are NOT placed in any wall cavity (hollow space within a wall), as this makes it very difficult to remove them should a load bar repair be necessary.

Please note that the edge of the platform must not touch the edge of the surrounding concrete otherwise there will be inaccurate weights recorded.

ONCE a WEEK a screwdriver or similar instrument must be run round the edge of the platform to make sure that there are no stones, chippings or other material touching both the platform and the edge of the concrete.

If situating the platform in a passageway it is advisable to locate it in the middle of the passageway where the horses walk naturally.

Once the carrying handles have been removed, and the indicator attached to the blue cables the platform is ready to weigh horses.

Photograph 1

Photograph 2

Neuadd Isaf, Penybont, Llandrindod Wells, Powys LD1 5SW, United Kingdom Tel and Fax +44 (0) 1939 220667   Mobile: +44  (0) 7774 783247

Email:  william@horseweigh.com    www.horseweigh.com